/*

+--------------------------------------------------------------------------+

| FlowMap: LUT-Based FPGA Technology Mapping Package (Release B 0.2)       |

+--------------------------------------------------------------------------+

| Copyright (C) 1991-1994 the Regents of University of California          |

+--------------------------------------------------------------------------+

| Restricted distribution only -- see "release.statement"                  |

+--------------------------------------------------------------------------+

| Author: Eugene Ding, VLSI CAD Lab, UCLA  CS Dept. <eugene@cs.ucla.edu>   |

+--------------------------------------------------------------------------+

| File Name:   flowmap.doc                                                 |

| Description: documentation for commands under flowmap/                   |

+--------------------------------------------------------------------------+

$Log: flowmap.doc,v $

# Revision 1.2  1997/02/03  23:05:04  peck

# Added -x switch describing level of effort.

#

*/

===================

<0> ACKNOWLEDGEMENT

===================

The FlowMap package is integreted into the SIS system and

uses many of the routines provided by SIS. The SIS system

was developed in UC Berkeley Electronic Research Lab.

====================================

<1> RELEASE AGREEMENT & CONTACT INFO

====================================

Please refer to "release.statement" under the top directory.

===========

<2> CONTENT

===========

This package works with SIS. To make it working with MIS, replace all

apearance of "SIS" and "sis" with "MIS" and "mis", and make proper

changes in the Makefile.

The existence of UC Berkeley Octtools (in particular, the mm package)

is assumed. If you donot have Octtools, change Makefile to remove the

reference links. (The code will be smaller, but slightly slower.)

Due to cross reference, you must install both dagmap/ and flowmap/

even if you only need commands under flowmap/. Note that the partitioning

of the commands into two subdirectories is due to the coding history and

is rather arbitrary.

This release contains programs primarily developed prior to March 1993,

with some bug fixes as late as Oct. 1994. More functions will be added

when they are stablized.

Makefile	-- SIS makefile for flowmap/

com_flmp.c	-- FlowMap command driver

dagflow.c	-- cut computation routines

dagflow.h	-- header file for cut computation routines

daglib.c	-- DAG manipulation package

dagtypes.h	-- header file for DAG manipulation package

dfmap.c		-- DF-Map duplication-free optimal mapper

dfmapaux.c	-- DF-Map auxiliaries (printing, preprocessing)

flmap2mt.c	-- multi-output CLB matching

flmapaux.c 	-- FlowMap auxiliaries: status printing

flmapdly.c      -- FlowMap auxiliaries for net-delay model

flmapini.c	-- FlowMap clustering initialization

flmapmsc.c	-- FlowMap support routines

flmaprlx.c	-- FlowMap support routines for -r option

flmapsyn.c      -- FlowMap support routines for -s option

(not in this release)

flwmatch.c	-- weighted graph matching package

flowmap.c	-- FlowMap k-LUT mapping / Flow-Pack area minimization

flowmap.doc	-- this file

flowmap.h	-- header file for flowmap programs

vismap.c        -- area-optimal mapper based on Woo's VISMap

(not in this release)

========================

<3> TECHNICAL REFERENCES

========================

J. Cong, Y. Ding, "An Optimal Technology Mapping Algorithm for Delay

Optimization in Lookup-Table based FPGA Designs," IEEE Trans. on CAD,

Vol. 13, No. 1, Jan. 1994, pp. 1-12.

J. Cong, Y. Ding, "On Area/Depth Trade-off in LUT-Based FPGA Technology

Mapping," IEEE Trans. on VLSI Systems,  Vol 2., No. 2, June 1994,

pp. 137-148.

J. Cong, Y. Ding, T. Gao, K. Chen, "LUT-Based FPGA Technology Mapping

under Arbitrary Net-Delay Models," Computers & Graphics,

Vol.18, No.4, 1994, 507-516.

J. Cong, Y. Ding, "Beyond the Combinatorial Limit in Depth Minimization

for LUT-Based FPGA Designs," Proc. 1993 IEEE/ACM Int'l Conf. on CAD,

Santa Clara, CA, Nov. 1993, pp. 110-114.

================

<4> INSTALLATION

================

Please refer to "installation.doc" in top directory.

=========

<5> USAGE

=========

The following SIS commands are added:

(1) FlowMap

-----------

flowmap [ -k <fanin_limit> ] [ -i <in_cluster_file> ] [ -t {m|M|F|n} ]

[ -d ] [ -r <relax_depth> [ -g <greediness> ] ]

[ -s <threshold> [ -e ] [ -x <synthesis_effort> ] ] [ -o <out_cluster_file> ]

Map a K-bounded network into a K-LUT network. ("dmig" command

in DAG-Map package may be used to transform a network into a

K-bounded one.) Strictly speaking, this is a partial mapper for

delay minimization, and it is usually followed by other mappers or

postprocessors for area minimization.

flowmap command consists of four steps: [a] read cluster, [b] form cluster,

[c] relax cluster, and [d] map cluster. some or all of the steps can be disabled

by default or by specifying command line options.

step [a] inputs pre-mapped clusters from a file. clusters are classified as

either "normal" or "permanent". normal clusters are kept unbroken (but may

be contained in larger clusters) in step [b] but are not honored in step [c].

permanent clusters are kept unaltered through steps [b] and [c].

note that there is no guaranty that a permanent cluster be implemented in

step [d] as a K-LUT (i.e. if the output is not required, it will be ignored).

The cluster file contains comment lines, white lines, and cluster lines.

a cluster line is of the form

<rootname><sep><size><sep><member_1><sep>...<sep><member_k>[<sep><flag>]

where <sep> is the field sepparator, defaulted to '|', <rootname> is the

name of the root node, <size> = k is the number of members of the cluster

(excluding the root), and <member_1> thru <member_k> are the node names of

the members, SORTED in reversed topological order (i.e. if node w is on a

path of node v to the root, then w appears before v in the list). the optional

<flag> can be 'n'/'N' or 'p'/'P', corresponding to normal or permanent cluster.

in fact, anything other than 'p',/'P' has the same effect as 'n'.

the default without <flag> is 'n'.

a white line consists of whitespace characters only (space and tab),

and are used to improve the readability. they are ignored.

a comment line has its first character to be <sep>, and is ignored, except

the special comment line of the form

<sep>==<new_sep>

which defines a new <sep> that will be in effect immidiately.

it is assumed that the node names can have any character other than <sep>

(and <sep> can be redefined as above to practically relax this constraint).

therefore, whitespace characters are NOT skipped. moreover, anything at the

end of a line after all expected fields are ignored, therefore comments

can be added there (after a <sep>). any line with none of the above formats

are considered an ill-defined cluster line. a line of the correct format

is read and parsed, and is examined to see if a valid cluster can be formed

for a node with no cluster already attached. if the check failed, a warning

about ill-defined cluster will be printed and that cluster ignored. If the

first three lines are all bad, it is assumed that the file is corrupted and

step [a] is aborted.

step [a] is disabled by default. command line option is needed to enable it.

step [b] mapps the network using the FlowMap algorithm, with variations.

it defines a cluster for each internal node, which will be the K-LUT rooted

at this node if this node is to be implemented with a K-LUT at all.

if step [a] is executed, the mapper will make sure that for any node with

a pre-defined cluster, the old cluster will be contained in the new one if

the old one is normal, and will be maintain as is if the old one is permanent.

mapping is based on flow computation to minimize delay under the above

constraint and other specified options.  by default, step [b] will be carried

out; command line option is required to disable it.

step [c] partially undoes mappings along non-critical paths to prepare a

partial mapping solution that satisfies given depth bound, and is favorable

to subsequent area minimization mapper(s). it makes sure that the

clusters defined by step [a] as permanent ones are not broken.

by default, step [c] is disabled; command line option is needed to enable it.

step [d] implements the clusters, produced in previous steps, with K-LUTs

wherever necessary, and sweeps out unnecessary ones. note that these "K-LUTs"

are not final, especially with step [c] is executed. they are rather

K-bounded complex gates to be mapped further. by default, step [d] is carried

out; using command line option can prevent this, and have the clusters

dumped into a file with the format specified for step [a] (with all new

clusters classified "normal"). for simplicity, a fixed separator '|' is used,

and the '|' character appearing in node names are replaced by '#' and reported

in warning comments.  the corresponding EQN output is also produced as comment

beginning with "|EQN|", which can be extracted as a mapping solution. however,

it may not be the same as the result of step [d], since in step [d] the network

is also repeatedly swept to (recursively) remove unnecessary nodes after the

clustering.

Options:

-k <fanin_limit>    specify fanin limit K. Default is 5.

-i <in_clstr_file>  specify the file containing the list of

pre-mapped clusters.

default is no cluster file.

-t 		    specify the method used.

m		    map based on min-volume min-cut

M		    map based on max-volume min-cut (default)

F		    map based on max-volume K-feas cut**

n		    don't do any new map. is useful only with -i

** heuristic algorithm for volume-maximization

-d		    assume arbitrary net-delay model. Delay is assigned

to the fanout net of each node. Such delay values

are recorded in sepcially dedicated field of node

records. Access to such records are currently via

delay data files. see the "fl_delay" item.

the default model is unit-delay model (net-delay=0)

-r <relax_depth>    relaxation mode. the mapping result will have

delay = (optimal_delay + relax_depth), and the

resulting network contains as many unsaturated

LUTs as possible -- i.e. mapping effort is made

only to meet the delay requirement. this intended

effect is for further area reduction. see the

"dfmap" entry.  the default <relax_depth> value

is 0, which implies relaxation along non-critical

path only. Default is NO relaxation (i.e. every

node has minimum delay).

-g <greediness>	    usable only when -r is used. Normally,

relaxation step will evaluate every possible

relaxation operation in each iteration and carry

out the best one. if a positive <greediness> of

G = 1 to 10 is given, only part (about (100-10G)% )

of operations are examined before carrying out

the operation. This may save some running time, and

not necessarily give inferior result (relaxation

is a heuristic anyhow). Default <greediness> is 0,

i.e. no greedy relaxation.

-s <threshold>      use functional decomposition to partially

resynthesize network if depth can be reduced.

use FlowSYN algorithm. <threshold> specifies

the maximum number of inputs the subnetwork

can have for it to be resynthesized (default is

10). In the current implementation, using -s

will disable -d (i.e. only unit-delay model

is supported) and -o (i.e. network will always

be changed into LUT network, and on cluster

file will be available).

-e                  used when -s is used, specifying that exhaustive

search of the best functional decomposition is

to be used. (WARNING: this is VERY expensive!)

-x		    used when -s is used, specifying the level of

effort given synthesis.  The acceptable values

are 0=minimum effort, 1=moderate effort,

2=considerable effort.

-o <out_clstr_file> specify the file that will contain the list of

mapped clusters. disabled by default. NOTE: this

option turns off the mapping, and leaves an

unaltered network.

(2) Flow-Pack

-------------

flowpack [ -k <fanin_limit> ] [ -i <in_cluster_file> ] [ -t {M|F|n} ]

[ -o <out_cluster_file> ]

Pack K-LUTs together when possible. this is actually a greedy version of

FlowMap with no concern on delay but all concerns on area, so options are the

same as above.  default is "-k 5 -t F". ("-t m" is not supported for lack of

use)

(3) DF-Map (part of FlowMap-r)

------------------------------

mffc_shrink [ -k <fanin_limit> ]

Collapse MFFCs with fanin size no more than <fanin_limit> (default is 5)

into a single node. This is a pre-processing step of DF-Map, and can improve

the efficiency of DF-Map by a large margin, so it is strongly recommended

to be applied before "dfmap".

dfmap [ -k <fanin_limit> ] [ -a | -d ] [ -t <threshold> | -o ]

[ -b ] [ -p <fanout_limit> ]

Optimal duplication-free K-LUT mapping. Part of FlowMap-r, used after

FlowMap with -r option.

Options:

-k <fanin_limit>	    fanin limit of LUT (default=5)

-a | -d			    mapping for area/depth minimization.

default is -a (for area)

-t <threshold> | -o	    optimal ( -o ) or  sub-optimal mapping.

with -o the program will look for the

optimal solution. WARNING: due to memory

management problem of MALLOC package,

-o option may require very large memory

and long execution time if MFFC is large

(#nodes>120), although this is rarely true.

with -t option the program will only check

the first <threshold> canditates. default

setting is "-t 50000" (this is more than

enough for most networks to be optimally

mapped).

-b			    secondary objective is to minimize the

input size of each LUT. by default,

secondary objective is to minimize depth

(area) when primary objective is to minimize

area (depth). with -b this becomes the third

objective. This option leaves better chance

for subsequent postprocessing to reduce

area by merging LUTs.

-p <fanout_limit>	    after dfmap, carry out a post-processing

to perform controlled duplication.

if a root LUT in current decomposition

can be merged into each fanout MFFC

without increasing the mapping cost of

the fanout MFFCs, it is duplicated and

merged. Only those with fanout size

no mote than <fanout_limit> will be

considered. -p turns -b on. Default

is no duplication.

(4) VISMap (area-optimal mapping)

vismap [ -k <fanin_limit> ] [ -t <threshold> ]

Compute area-optimal mapping using N. Woo's VISMap algorithm. This routine

is implemmented for comparative study only. Due to its exhaustive search

nature, it is very slow.

Options:

-k <fanin_limit> 	    specify the LUT fanin limit (default 5).

-t <threshold>		    instead of finding the optimal mapping,

instruct the algorithm to stop when a

solution of no more than <threshold>

LUTs are found. By default optimal

mapping will be computed. Note: specifying

a very small <threshold> will actually

yield the optimal mapping.

(5) Multi-output CLB Matching

match_two [ -l <left_lut_size> ] [ -r <right_lut_size> ]

[ -c <combined_clb_size> ] [-w <nominal_weight_percentage> ]

[-v | -f <outfile> ]

Compute the best matching on an LUT network, which gives a two-output CLB

mapping solution. Each pair will contain a "left LUT" of size <= left_lut_size

and a "right LUT" f size <= right_lut_size, which can be put into the same

CLB of size = combined_clb_size. Unpaired singletons are CLBs with unused

left or right LUT.

It is possible that instead of the maximum cardinality matching, a weighted

matching is computed to reflect certain considerations in pairing up the

LUTs (e.g. routability, etc.). Currently, a simple weight function

w(u, v) = computed_weight(u, v) + nominal_weight

is used, where the "nominal weight" is actually a "non-weight" factor to

tune down the weighting factor. a parameter w specifying the "percentage" of

nominal weight can be supplied. by changing w (0<=w<=100), the significance

of the weight can be changed.  with w=100, maximum cardinality matching is

guaranteed and weight is only used to break ties. with w=0, cardinality is

completely ignored. if w is not specified, normal cardinality matching

is carried out (note this may be different from w=0), which is faster.

when verbose mode (-v or -f) is selected, the list of CLBs will be printed.

There are three types of CLBs:

(1) full CLB with 2 LUTs, in the format ( Left_LUT , Right_LUT )

(2) half-full CLB with one LUT, in the form ( Left_LUT , () ) or

( () , Right_LUT )

(3) single LUT CLB, in the form of ( LUT )

Example of output format:

--------------------------------------------

#pi=10  #po=6   #nodes=153      #clb=82

#0001: ( [380] , [125] )

#0002: ( [123] , [124] )

#0003: ( [585] , () )

#0004: ( () , [587] )

#0005: ( [507] , () )

#0006: ( [295] , [506] )

#0007: ( [504] , [374] )

#0008: ( [497] , () )

#0009: ( [495] )

#0000: ( [490] , [273] )

... ... ... ...

--------------------------------------------

Options:

-l <left_lut_size>	input size limit of left LUT. default 4.

-r <right_lut_size>	input size limit of right LUT. default 4.

-c <combined_clb_size>	input size limit of the CLB. default 5.

this is the fanin limit of an unmatched LUT.

-w <nominal_weight_percentage> 	the percentage of added nominal weight.

default is off (i.e. no weight is considered).

allowed value 0 to 100.

-v			verbose output matching list to standard output.

default is silent output (i.e. only CLB count).

-f <outfile>		verbose output matching list to file.

Note that the default set reflects mapping for Xilinx XC3000 series CLBs, where

paired LUTs usually share up to 3 inputs, so weight can be turned off.

For Xilinx XC4000 series CLBs, use -l 4 -r 4 -c 8 -w xx and decrease xx if you

encounter routing problem (or use 'match_4k' for probably better result).

(6) Net-Delay Data Access

-------------------------

fl_delay { -i [ <file> ] | -o [ <file> ] | -p [ -v ] }

Access net-delay data when network is mapped under net-delay model.

the delay file has the following format:

<network_name> <netowrk_size> <pi_delay> <po_delay> <lut_delay>

<node_name> <node_fanout_net_delay>

<node_name> <node_fanout_net_delay>

... ... ... ...

<node_name> <node_fanout_net_delay>

(i.e. whitespace is used as field separator, and no comment, redefinable

separator, empty lines, etc. are supported.)

note that PO nodes are attached when the network is loaded, they are not

included (a PO node does not have fanout anyway).

Options:

-i [ <file> ]		    read delay values from <file>  and assign

to each net. if no <file> parameter, the

default is <network_name>.dly. ***

-o [ <file> ]		    write net-delay values to <file>. default

is <network_name>.dly. ***

-p [ -v ]		    print out the overall delay of the network.

with -v option, a critical path will also

be printed. ***

*** command "fl_delay -o|-p" assumes that the delay values are properly

assigned to nodes(nets). If this is not true, the result can be meaningless.

"fl_delay -i" requires that the name of network and the nodes are matched

strictly.  it's a good approach to use "fl_delay -o" to get a template

for the delay data file.

(7) Model-Based Delay Assignement

---------------------------------

_fl_delay_via_fo  { -l | -r | -w } <model_file>

Assign net-delays according to delay model ( -l ); read/write delay model

( -r/-w ). Generic models are coded into program; <model_file> provides

parameters. when -w option is specified, parameters are keyed in upon

prompt. when -r is specifed, delay models are read and printed (no

assignment). This part is NOT for general use. Please see "flmapaux.c"

for details.

_fl_delay_nominal_predict <a> <b> <c>

Assign net-delays according to nominal predict delay model.

When <a> is positive, <a> is the delay of clb;  <b> is the scale factor

of the net delay; <c> is the scale factor of the net delay adjustment.

When <a> is 0, the clb delay will be the current one. When <a> is

negative, the clb delay will be set to (M - <a>), where M is the largest

net delay (this is used to force a larger clb delay than net delay). net

delay is computed as <b>|output(v)| + <c>( |output(output(v))| - |output(v)| ).

iob delay is set to 0.  This part is NOT for general use. Please see

"flmapaux.c" for details.

(8) Printing Commands

---------------------

fl_print_mffc [ -v ]

Print MFFC decomposition of the network. -v option will print the root,

members, size, and fanin size of each MFFC; without -v the members are

not printed. Printing format is bad (sorry).

fl_nstatus [ -d ] [ -s | -v ]

Print network status. With -d option non-unit delay model is assumed; the

net delay values must have been properly attached to obtain meaningful

results. by default unit delay model is assumed. Will print #inputs, #output,

#nodes, overall delay, and I/O size statistics for the network. With -s

option, I/O status will include the count of nodes for each fanin and fanout

size. With -v option, in addition to the effect of -s option, for each node

the name, indegree, outdegree, slack (w.r.t the overall network delay),

fanin and fanout nodes are also printed (this format is usually hard

to read on screen; better redirect to a file).

(A) functional interface for delay access

-----------------------------------------

4 functions are provided for users that want to write SIS functions to set/get

net delay values. These should be regarded as the only program interface;

any other direct access to FlowMap auxiliary structures may rersult in

unexpected side-effect.

extern int _fl_set_clb_delay(di, dt, dc)

int di, dt, dc;

set PI/PO/LUT delay to be di/dt/dc. Only valid values (di>=0, dt>=0; dc>0)

are set; invalid values are ignored. returns # of values CHANGED.

extern int _fl_get_clb_delay(di, dt, dc)

int *di, *dt, *dc;

get PI/PO/LUT delay from di/dt/dc. If not all three are wanted, pass NULL

to indicate unwanted values. returns number of values provided.

extern int _fl_get_net_delay(node)

node_t *node;

returns the fanout net delay of node, assuming that it has been properly set.

extern int _fl_set_net_delay(node, value)

node_t *node; int value;

set fanout net delay of node to value. If value<0, it is not set and function

returns -1; otherwise returns value.