genlib.doc

Richard Rudell

10/28/87

12/29/87 Rev. 1

A cell is specified in the following format:

GATE <cell-name> <cell-area> <cell-logic-function>

<pin-info>

.

.

<pin-info>

<cell-name> is the name of the cell in the cell library.  The resulting

net-list will be in terms of these names.

<cell-area> defines the relative area cost of the cell.  It is a floating

point number, and may be in any unit system convenient for the user.

<cell-logic-function> is an equation written in conventional algebraic

notation using the operators '+' for OR, '*' or nothing (space) for AND,

'!' or ''' (post-fixed) for NOT, and parentheses for grouping.  The

names of the literals in the equation define the input pin names

for the cell; the name on the left hand side of the equation defines

the output of the cell.  The equation terminates with a semicolon.

Only single-output cells may be specified.

Also, the actual factored form is significant when a logic function has

multiple factored forms.  In principle, all factored forms could be

derived for a given logic function automatically; this is not yet

implemented, so each must be specified separately.  Note that factored

forms which differ by a permutation of the input variables (or by De

Morgan's law) are not considered unique.

Each <pin-info> has the format:

PIN <pin-name> <phase> <input-load> <max-load>

<rise-block-delay> <rise-fanout-delay>

<fall-block-delay> <fall-fanout-delay>

<pin-name> must be the name of a pin in the <cell-logic-function>,

or it can be * to specify identical timing information for all pins.

<phase> is INV, NONINV, or UNKNOWN corresponding to whether the

logic function is negative-unate, positive-unate, or binate in

this input variable respectively.  This is required for the separate

rise-fall delay model.  (In principle, this information is easily

derived from the logic function; this field may disappear in the future).

<input-load> gives the input load of this pin.  It is a floating point

value, in arbitrary units convenient for the user.

<max-load> specifies a loading constraint for the cell.  It is a

floating point value specifying the maximum load allowed on the output.

<rise-block-delay> and <rise-fanout-delay> are the rise-time parameters

for the timing model.  They are floating point values, typically in

the units nanoseconds, and nanoseconds/unit-load repsectively.

<fall-block-delay> and <fall-fanout-delay> are the fall-time parameters

for the timing model.  They are floating point values, typically in

the units nanoseconds, and nanoseconds/unit-load repsectively.

All of the delay information is specified on a pin-by-pin basis.  The

meaning is the delay information for the most critical pin is used

to determine the delay for the gate.

---------------------------------------------------------------------------

Example 1 of <cell-logic-function>

O = !(!I1 * I2);

is legal and defines a NAND gate with 1 inverted input; it could also

be specified as

O = I1 + !I2;

There is no advantage or disadvantage to either representation; only 1

need be given.

---------------------------------------------------------------------------

Example 2 of <cell-logic-function>

O = !((I1 * I2) + !(I3 + I4));

is incorrectly specified.  It must be re-written as either:

O = !((I1 * I2) + (!I3 * !I4));

or

O = (!I1 + !I2)*(I3 + I4);

Again, there is no advantage or disadvantage to either representation as

they differ only by De Morgan's law.  Only 1 need be given.  Note that

there are no other factored forms for this equation.

---------------------------------------------------------------------------

Example 3 of <cell-logic-function>

O = a*b + a*c + b*c;

is one specification of a majority gate.  Another possible specification

(which IS considered different) is:

O = a*(b + c) + b*c;

Any permutation of the input variables does not provide a new specification;

hence, these are the only two useful representations for a majority

gate logic function.  Both should be provided to the technology mapping

program, until further notice.