XILINX

Description:

This is a package to optimize the Boolean network and map it onto

the Xilinx Programmable Gate Array architecture.(ref: Xilinx, the

Programmable Gate Array Data Book, Xilinx Corporation)

void

merge_node(network, MAX_FANIN, MAX_COMMON_FANIN, MAX_UNION_FANIN)

network_t *network;

int MAX_FANIN, MAX_COMMON_FANIN, MAX_UNION_FANIN;

For mapping onto Xilinx architecture.

Exploits the fact that a CLB can realize two functions also,

provided the two functions satisfy the constraints as listed below:

Each of the two functions should not have fanin more than MAX_FANIN,

they should not have more than MAX_COMMON_FANIN many common fanins,

and the union of their fanins should not exceed MAX_UNION_FANIN.

For xilinx XC3020 the numbers are MAX_FANIN = 4, MAX_COMMON_FANIN = 3

and MAX_UNION_FANIN  = 5.

The problem is formulated as an integer program, and solved using package

Lindo. If Lindo package is not found in the path, the program exits.

If found, the routine lists the pairs of nodes that were merged.

Does not change the network.

void

partition_network(network, n, bincov_heuristics)

network_t       *network;

int             n;

int             bincov_heuristics;

For mapping onto xilinx architecture.

The "network" should have all intermediate

nodes with fanin less than or equal to "n".

Uses Mathony's binate covering algorithm to

minimize the number of nodes in the network.

Uses different heuristics to solve the covering

problem- "bincov_heuristics" ( 0, 1, 2 or 3) is

a flag that specifies which heuristic should be

employed, as described below:

-  0 (exact),

-  1 (Luciano's heuristic),

-  2 (For large examples),

-  3 (automatically switches between 0 and 2)

void

karp_decomp_network(network, support, one_node, user_node)

network_t *network;

int support;

int one_node;

node_t *user_node;

Uses Karp_Roth disjoint decomposition to decompose recursively

nodes of the network having fanin greater than

"support". If "one_node" is 1, it just decomposes the

"user_node", else it decomposes all the nodes of the

network whose fanin is greater than "support".

In the present implementation, the

first decomposition found is selected and no attempt

is made to have the best decomposition.

int

split_network(network, size)

network_t *network;

int size;

Does a decomposition of the network so that all nodes have a fanin

less than the value of size. Uses kernel extraction and AND-OR

decomposition. Returns one on success.

int

part_network(network, size)

network_t *network;

int size;

Attempts to partition the network locally so that each node has

fanin constraint equal to size. Does collapsing depending on

the number of nodes/edges of nets that have to be duplicated.

ACTEL

Description

Routine to find an optimal mapping to the Actel architecture. The input

is the Boolean network and the output is a netlist and the block count.

(ref: An Architecture for Electrically Configurable Gate Arrays , Gamal et al.

IEEE Jour. of Sol. St. Cir. April 1989 p394-398)

network_t *

act_map_network(network, HEURISTIC_NUM, NUM_ITER, FANIN_COLLAPSE, GAIN_FACTOR,

DECOMP_FANIN, DISJOINT_DECOMP, QUICK_PHASE, cost_table)

network_t *network;

int  HEURISTIC_NUM, NUM_ITER, FANIN_COLLAPSE, DECOMP_FANIN, DISJOINT_DECOMP,

QUICK_PHASE;

float GAIN_FACTOR;

st_table *cost_table;

This procedure synthesizes the given circuit onto actel

architecture. The original network "network" is lost.

It uses tree-mapping approach to

cover the subject graph with the pattern-graphs.

The pattern graphs are hardwired into the code

and so no library is to be read in.

Subject graph and pattern-graphs are in terms of

2-1 muxes. Subject graph is constructed for each intermediate

node of the network. Either an OBDD (Ordered reduced BDD)

and/or a BDD (Binary Decision Diagram) is constructed for each

such node. After entire network is mapped, an iterative_improvement

phase may be entered.

HEURISTIC_NUM (1, 2, 3, 4) specifies which one of the two

subject_graphs would be constructed:

HEURISTIC_NUM = 1 => OBDD

HEURISTIC_NUM = 2 => BDD

HEURISTIC_NUM = 3 => program decides which one to construct.(default).

HEURISTIC_NUM = 4 => both are constructed and the one with

lower mapped cost is selected. Gives the best

result, but typically takes more time.

NUM_ITER equals the maximum number of iterations

to be performed in the iterative_improvement phase. Each such

iteration involves a good_decomposition followed by a partial_collapse

routine.

If the iterative_improvement phase is entered, in the partial_collapse

routine, only those nodes are considered as candidates

for partial_collapse which have fanin no more than FANIN_COLLAPSE.

The good_decomposition routine is called only for those nodes

which have fanin greater than or equal to DECOMP_FANIN.

The program enters the next iteration only if gain in the present

iteration is at least (present_cost * GAIN_FACTOR).

The cost and gain are measured in terms of the number of basic blocks

used for the network.

If DISJOINT_DECOMP is non-zero, disjoint_decomposition routine

is invoked on the network at the very start. This may help in

getting big gains in partial_collapse routine later. Also, big

nodes are decomposed, potentially reducing the final cost.

If QUICK_PHASE is non-zero,  the program, before exiting,

enters an act_quick_phase routine, which greedily

finds out if it is beneficial to implement the node in

negative phase.

The cost_table should be initialized by

st_init_table() before calling this routine. After the routine, it has

a subject graph for each node. Call free_cost_table() to free all the

associated storage. Also it is a serious error to call act_map_network()

with negative flags.

void

free_cost_table(cost_table)

st_table *cost_table;

Frees the associated storage associated with the

cost_table. This space was allocated by the act_map_network()

routine.