/* In the following description the words "coeff" and

"thresh" have a special meaning.

"thresh" is the threshold for the creating an epsilon-

network. Nodes with a slack within "thresh" of the

most critical signal are candidates for speedup.

"coeff" selects the relative importance of area. If set

to 1 it tries to recover as much area as possible. If

0 then area increase is ignored. Intermediate values can

be used to obtain a area/speed  tradeoff.

*/

void

speed_up_loop(network, thresh, coeff, dist, model, flag)

network_t **network;

delay_model_t model;

double thresh, coeff;

int dist, flag;

Speed-up the network and keep the best network seen till

now. Exits the program if a serious error occurs.

"dist" is the distance for collapsing. A fast value is 3

and a good value is 6.

"model" is the delay model used for computing delays during

the speedup. The default is DELAY_MODEL_UNIT_FANOUT.

"flag" = 1 produces an output to "misout" describing how the

delay changes at each iteration. Flag = 0 is the silent mode.

void

speed_up_node(network, node, coeff, model, flag)

network_t *network;

node_t *node;

double thresh, coeff;

delay_model_t model;

int flag;

Speedup the node f belonging to the network. The node f is

modified in place by a 2-input nand gate decomposition.

ASSUMES that a delay trace using the delay_model "model" has

already been done so that the arrival times at the inputs of

node f are valid.

"flag" MUST be set to 0.

Exits the program if there is a serious error.

array_t *

speed_decomp(f, coeff, model)

node_t * f;

double coeff;

delay_model_t model;

Returns an array of nodes representing the node f as a

2-input nand representation. The array contains nodes of

type NODE_PO, NODE_PI and internal nodes. Only the

internal nodes should be used by the calling routine.

This is a manifestation of the fact that the nodes in the

returned array are not part of the network, but nodes in

the array have the same fanins as node f. The first

internal node is the root node and the node f can be

replaced by this node. The remaining nodes can be simply

added to the network.

Returns NIL(array_t) if there has been an error.

/*

Routines to improve circuit performance by adding buffers.

*/

int

buffer_network(network, limit, trace, thresh, single_pass, do_decomp)

network_t *network;

int limit;

int trace;

double thresh;

int single_pass;

int do_decomp;

This routine operates on an entire network and will find

out appropriate sites to add buffers. If the network is mapped,

the "mapped" delay model will be used else the "unit-fanout"

model is used. The routine will iterate over different

cutsets of the critical paths (except when "single_pass" = 1).

If the required time is set for the primary outputs the algorithm

will try to meet them, otherwise it will buffer so as to reduce

the delay thru the circuit.

The critical paths are determined by the parameter "thresh".

The paths with a slack value within "thresh" of the smallest

slack are deemed to be critical.

If the "trace" parameter is set to 1, the delay data will be

printed to misout as the algorithm proceeds.

"limit" specifies the acceptable fanout limit for a gate.

Nodes with fanout less than this are not candidates for buffering.

If "do_decomp" parameter is specified, the routine will

also attempt the decomposition of complex gates in an effort to

increace performance.